AI-Assisted Floor Plan Layout Generator

Phase 1: Rules & Preferences Research

Philippine Single-Detached, Mid-Market Homes

Prepared for AI Solution Maven

24 May 2026  ·  Draft v0.1

Primary source: PD 1096 — National Building Code of the Philippines

# Contents

# 1. Executive summary & recommendation

This document is the foundation layer for an AI-assisted floor plan layout generator targeting single-detached, mid-market houses in the Philippines. It separates the rules that govern a layout into two clearly distinct categories, because they behave very differently inside a generation or validation engine.

Hard constraints are statutory minimums drawn from PD 1096, the National Building Code of the Philippines (NBCP). A layout that violates any of these is illegal and cannot be permitted, so the engine must treat them as non-negotiable filters. Soft preferences are design targets drawn from common Philippine residential practice — preferred room dimensions, room adjacencies, climate orientation and cultural spaces such as the dirty kitchen and service area. These are what make a layout feel right for a Filipino household, and the engine should optimize toward them while being free to trade them off.

Recommended approach: build the knowledge base first (this phase), encode it as the machine-readable ph_floorplan_rules.json file that accompanies this report, then build the validation engine before the generator. The validator is the lower-risk, higher-certainty component — it reuses exactly the same rule set, gives you a testable contract for what “compliant” means, and becomes the scoring function the generator optimizes against.

## Suggested phasing

- Phase 1 (this deliverable): structured rules + preferences knowledge base, with code citations and a hard/soft split.

- Phase 2: code-validation engine — takes a candidate layout (rooms as rectangles/polygons + lot) and returns errors, warnings, and suggestions against the rule set.

- Phase 3: 2D generator — places rooms on the lot subject to the hard constraints and optimizes the soft objectives, then runs every candidate through the Phase 2 validator before presenting it.

# 2. Scope & legal basis

Segment: single-detached, mid-market homes. Under the NBCP these fall under Group A (Residential Dwellings), so the governing dimensional rules are PD 1096 Sections 701, 704–708, and 801–811.

Two adjacent regimes are deliberately out of scope but recorded for later. BP 220 sets smaller minimums (18–22 sqm minimum dwelling area) and applies only to economic/socialized housing — it does not govern this mid-market segment. The NBCP Implementing Rules & Regulations (IRR, 2004) and the relevant LGU zoning ordinance refine setbacks, yards, road-right-of-way and parking ratios; PD 1096 gives only the floor (e.g. a flat 2 m from the property line). Anywhere this report states a setback finer than 2 m, it is flagged as “IRR/LGU — to confirm.”

# 3. Hard constraints (PD 1096)

These are statutory minimums. In the engine they act as filters: any candidate layout that breaches one is rejected (severity = error).

## 3.1 Minimum room sizes (Sec. 806)

The single most important table for layout generation. These are absolute floors — no generated room of the given type may be smaller in either area or least horizontal dimension.

| Room type | Min. area | Min. least dimension | Source |
| --- | --- | --- | --- |
| Rooms for human habitation (bedroom, living, dining, etc.) | 6.00 sqm | 2.00 m | Sec. 806(1) |
| Kitchen | 3.00 sqm | 1.50 m | Sec. 806(2) |
| Bath & toilet | 1.20 sqm | 0.90 m | Sec. 806(3) |

## 3.2 Light, ventilation & ceiling heights (Sec. 805, 808–810)

| Rule | Requirement | Source |
| --- | --- | --- |
| Window free area (no artificial ventilation) | ≥ 10% of room floor area, opening to court / yard / street / alley | Sec. 808 |
| Ceiling height — habitable, natural ventilation | ≥ 2.70 m | Sec. 805(a) |
| Ceiling height — habitable, artificial ventilation | ≥ 2.40 m | Sec. 805(a) |
| Ceiling height — multi-storey, 1st / 2nd storey | ≥ 2.70 m / ≥ 2.40 m | Sec. 805(a) |
| Headroom — succeeding storeys | ≥ 2.10 m | Sec. 805(a) |
| Air space per person, habitable rooms | ≥ 14.00 cubic m | Sec. 807(3) |

## 3.3 Interior rooms — courts & vent shafts (Sec. 804, 809)

This is how a room without an exterior wall (e.g. an interior toilet) is made legal. It is also a major driver of layout topology: every habitable room must ultimately reach light and air.

| Element | Requirement | Source |
| --- | --- | --- |
| Court — minimum horizontal dimension | ≥ 2.00 m | Sec. 804(a) |
| Inner court connection | Passageway ≥ 1.20 m wide, or a door through room(s) | Sec. 804(b) |
| Vent shaft — cross-section | ≥ 0.10 sqm per metre of height; never < 1.00 sqm | Sec. 809(a) |
| Vent shaft — least dimension | ≥ 600 mm | Sec. 809(a) |

## 3.4 Dwelling, lot & circulation (Sec. 704, 708)

| Rule | Requirement | Source |
| --- | --- | --- |
| Lot occupancy — inside lot / corner lot | ≤ 80% / ≤ 90% | Sec. 708(a), 802 |
| Distance from property line | ≥ 2.00 m | Sec. 708(a) |
| Eaves over required windows from side/rear line | ≥ 0.75 m | Sec. 704(a) |
| Access | Must adjoin a public space / yard / street on ≥ 1 side | Sec. 704(a) |
| Stairs | Clear width ≥ 750 mm; riser 200 mm; run ≥ 200 mm | Sec. 708(h) |
| Entrance / exit | ≥ 1 entrance and ≥ 1 exit | Sec. 708(i) |
| Sanitation | ≥ 1 sanitary toilet; washing + drainage facilities | Sec. 708(c), 901–903 |

# 4. Preferred (target) dimensions

The code minimums are far below what mid-market buyers expect — a 2.0 m × 3.0 m bedroom is legal but uncomfortable. The table below gives the target ranges the generator should aim for, gathered from common Philippine residential practice. These are soft objectives (severity = suggestion if missed).

| Room | Code minimum | Comfortable minimum | Preferred range |
| --- | --- | --- | --- |
| Bedroom (standard) | 6.0 sqm / 2.0 m | 2.5 × 3.0 m | 3.0 × 3.5 m (9–12 sqm) |
| Master bedroom | 6.0 sqm / 2.0 m | 3.0 × 3.5 m | 3.5 × 4.5 m (12–20 sqm) |
| Living room / sala | 6.0 sqm / 2.0 m | 3.5 × 5.0 m | 4.5 × 6.0 m (16–27 sqm) |
| Dining area | 6.0 sqm / 2.0 m | 3.0 × 3.0 m | 3.0 × 4.0 m (10–16 sqm) |
| Kitchen (main) | 3.0 sqm / 1.5 m | 2.0 × 2.0 m | 2.5 × 3.5 m (6–12 sqm) |
| Dirty kitchen | 3.0 sqm / 1.5 m | 2.0 × 2.0 m | 2.0 × 3.0 m (4–9 sqm) |
| Bath & toilet | 1.2 sqm / 0.9 m | 1.0 × 2.0 m | 1.8 × 2.1 m (3–4.5 sqm) |
| Service / laundry area | — | 1.5 × 3.0 m | 4–9 sqm |
| Lanai | — | 2.0 × 3.0 m | 6–16 sqm |
| Carport (sedan / SUV) | — | 2.5 × 5.0 m | 3.0 × 5.5 m; double 5.5 × 5.5 m |

Circulation & openings (soft): hallway clear width 0.90 m minimum, 1.0–1.2 m preferred; doors 800 mm general / 700 mm bath / 900 mm main entry; preferred window area ~15% of floor (vs the 10% legal floor); ceilings 2.7–3.0 m for passive cooling.

# 5. Philippine design preferences & spatial logic

These are the patterns that make a plan read as a Filipino home rather than a generic one. They should be encoded as adjacency rules and zone assignments the generator optimizes toward.

## 5.1 Three-zone organization

Filipino homes are zoned into public, private and service areas. Keeping these coherent — and buffered from one another — is the backbone of a good layout.

- Public zone: living room (sala), dining, foyer, lanai, guest powder room. Sits on the arrival side, adjacent to the main entry.

- Private zone: bedrooms clustered together as sleeping quarters, buffered acoustically and visually from public and service zones.

- Service zone: kitchen, dirty kitchen, service/laundry area, maid's room, carport. The kitchen–dining–service “loop” supports multitasking without congestion and is kept discreet from living areas.

## 5.2 Signature Filipino spaces

- Dirty kitchen (service kitchen): a near-universal expectation. Heavy, aromatic frying/grilling happens here, keeping the main kitchen clean. Placed at the rear, adjacent to the main kitchen, well ventilated and often semi-outdoor with direct exterior access.

- Service area: laundry, washing and drying — discreet from living zones, benefits from daylight for drying, typically grouped with the dirty kitchen at the rear.

- Maid's / helper's room: when included, placed beside the kitchen on the ground floor for service convenience. As a habitable room it must still meet the 6.0 sqm / 2.0 m minimum.

- Lanai: a covered outdoor extension of the living/dining area for gatherings and al fresco dining, opened up with wide sliding doors to the rear yard.

## 5.3 Tropical orientation & ventilation

- Prevailing winds: north-easterly (amihan, Oct–Mar) and south-westerly (habagat, Apr–Sep). Orient living and sleeping openings to catch these for cross-ventilation.

- Minimize large unshaded openings on the east and west facades to cut morning/afternoon solar gain; favor north and south exposures for habitable rooms.

- Use service spaces (carport, storage, service area) as thermal buffers on the hot east/west facades.

- Aim for openings on at least two walls of each habitable room to enable a cross-breeze; passive cooling can cut energy use by roughly 20%.

- Provide roof overhangs / eaves to shade sun-exposed openings.

# 6. The constraint model

The accompanying ph_floorplan_rules.json file encodes everything above in a form both the validator and the generator can read. The model has three parts:

- global_constraints — lot, setback, ceiling, stair, egress, sanitation, light/ventilation, court and vent-shaft rules that apply to the whole dwelling.

- room_catalog — one entry per room type, each carrying its hard minimums (with code citation), soft preferred dimensions, and adjacency hints (prefer_near / avoid_near / zone / cluster).

- zoning_model & orientation_model — the public/private/service grouping rules and the tropical-orientation heuristics, expressed as soft rules.

Every rule carries a source field. Statutory rules cite the PD 1096 section; preference rules are marked “PH residential practice”; and anything needing the IRR or LGU ordinance is explicitly flagged “IRR/LGU — to confirm,” so nothing unverified is silently treated as law.

## 6.1 Severity levels for the validator

| Severity | Meaning | Engine behaviour |
| --- | --- | --- |
| Error | Violates a hard (statutory) constraint | Reject / block the layout; must be fixed |
| Warning | Violates an IRR/LGU rule still to be confirmed per project | Flag for human review |
| Suggestion | Outside a soft preference target | Legal but sub-optimal; lower the layout's score |

# 7. Recommended prototype architecture

Both target components — the 2D generator and the validation engine — read the same rule file. Build the validator first.

## 7.1 Validation engine (build first)

Input: a candidate layout (room polygons/rectangles with types + the lot boundary). The engine checks each room and the whole dwelling against the rule set and returns a structured list of errors, warnings and suggestions. Because it is deterministic and testable, it doubles as the acceptance test suite and as the scoring function for the generator.

## 7.2 2D generator (build second)

A constraint-satisfaction + optimization problem. Hard constraints define the feasible region (room minimums, setbacks, occupancy cap, light/ventilation access); soft preferences define the objective to maximize (preferred dimensions hit, good adjacencies, sensible zoning, favorable orientation). Practical starting techniques include a rectangular space-partitioning / treemap-style subdivision of the buildable envelope, or a grid/graph placement seeded by the zoning model, with every candidate run through the validator before it is shown.

Suggested first milestone: hard-code a fixed room program (e.g. 3 bedrooms, 2 T&B, living, dining, kitchen, dirty kitchen, service area, carport) on a typical 120–200 sqm lot, generate a handful of candidate layouts, and validate each. This proves the rule file and the validator end-to-end before investing in a sophisticated solver.

# 8. Caveats & next steps

- Confirm the IRR & LGU layer. Setbacks finer than 2 m, parking ratios and road-right-of-way come from the NBCP IRR (2004) and the local zoning ordinance, which vary by city/municipality. These should be loaded per project and currently sit as flagged placeholders.

- Validate preferred dimensions with a local architect. The target ranges reflect widely published practice but should be reviewed against your own design standards and target buyer profile before launch.

- Decide the room program model. How will users specify what rooms they want (fixed templates vs free program)? This shapes the generator's input contract.

- Plan for fire/structural rules later. Fire-resistive and structural provisions (Chapters 4–6) matter for construction documents but are largely not layout-shaping; defer them past the prototype.

- Confirm units & lot assumptions. The model is metric throughout. Typical mid-market lots run ~120–200 sqm; confirm the lot-size band you are targeting.

# 9. Sources

Primary legal source: PD 1096 — National Building Code of the Philippines (uploaded text), Sections 701, 704–708, 801–811, 901–903.

- National Building Code of the Philippines (PD 1096) — full text

- Standard Residential House Dimensions (Philippines)

- Ideal House Size for a Filipino Family — Minami Residences

- BP 220: Housing and Land Use — VIZCODE

- Revised IRR of BP 220 (2008) — DHSUD

- Tropical Architecture: Climate Design in the Philippines — Fulgar Architects

- Climate-Responsive House Designs — Philippine House Designs

- How Filipino Homes Maximize Space with Smart Design — PHINMA Properties

- Creative Dirty Kitchen Ideas for Filipino Homes — Coohom